BTEX vapour biodegradation rates in the vadose zone: initial estimates

Title: 16 pt bold; left justify

(Skip 2 lines)

G. B. DAVIS, T. R. POWER, D. BRIEGEL & B. M. PATTERSON

Author name(s): 12 pt bold caps; use “&” before last name; left justify; indent 2.54 cm

Centre for Groundwater Studies, CSIRO Land and Water, Private Bag, PO Wembley, Western Australia 6014, Australia

Full postal address: 9 pt italic, left justify

greg.davis@per.clw.csiro.au

E-mail address of corresponding author: 9 pt; all lower case

(Skip 1 line)

Abstract Field investigations provided in situ estimates of biodegradation rates of BTEX (benzene, toluene, ethylbenzene, xylene) vapours in a gasoline-contaminated vadose zone. The soil was predominantly a leached, low-organic carbon content sand with variably cemented layers and a water table about 3.0 m below ground. Soil gas profiles indicated that oxygen consumption, carbon dioxide production and hydrocarbon vapour biodegradation occurred in a thin, shallow zone of the soil profile, typically within the top 0.8 m of the soil profile. Modelling of static oxygen concentration depth-profiles gave average oxygen consumption rates of 28–43% day-1. These gave average hydrocarbon vapour biodegradation rates of 110–170 mg-hexane l-1 day-1.

Abstract: 11 pt; fully justify; word “Abstract” in bold

Key words sand; steady-state; field site; Perth; depth profile; oxygen; multiport sampler; in situ

(Skip 2 lines)

Key words: 9 pt; fully justify; can make 2 lines;

“Key words” in bold

Level 1 heading:12 pt bold caps; left justify; skip 2 lines before 1 line after heading

INTRODUCTION AND BACKGROUND INFORMATION

(Skip 1 line)

Zones of biodegradation of BTEX (benzene, toluene, ethylbenzene, xylene) vapours were identified within a gasoline-contaminated vadose zone at a field site in Perth, Western Australia. These were related to soil physico-chemical aspects, and biodegradation rate estimates were calculated from steady-state BTEX and soil gas profiles.

Do not indent 1st paragraph after level 1 and level 2 headings

Groundwater at the site was approximately 3 m below ground, with very high concentrations of the BTEX range of compounds (Johnston et al., 1997). Gasoline product thicknesses of 10–15 cm were measured in open boreholes. Johnston et al. (1997) described the shallow stratigraphy in the vicinity of the site, and noted coarse- and medium-sand sized layers.

Indent subsequent paragraphs 0.7 cm

A core recovered from the site (Fig. 1) showed high hydrocarbon concentrations 2.2–3 m below ground. Benzene concentrations were low relative to the xylenes and other compounds. Soil air-filled porosities were 0.3–0.45 m3 m-3. Bulk densities were 1.3–1.45 g cm-3. Soil moisture contents were typically 0.07–0.09 m3 m-3 at the top of the profile.

Note layout of units—negative powers used not “/”

(Skip 2 lines)

MAJOR GAS AND BTEX VAPOUR CONCENTRATION PROFILES

(Skip 1 line)

A multiport borehole VMP1 was installed to sample soil gas depth profiles. VMP1 consisted of a bundle of 2 mm i.d. and 3 mm o.d. nylon tubes, with ends covered with nylon mesh to avoid clogging by soil media. Details are given in Davis et al. (1997).

Oxygen concentrations decreased from 20–21% at ground surface to approximately 1% at 0.5–0.75 m below ground (Fig. 2). Carbon dioxide concentrations increased from below detection levels (0.1%) at the ground surface to approximately 12.5% over this same depth interval. TPH (total petroleum hydrocarbon) and BTEX concentrations were non-zero to within 0.05 m and 0.25 m of the ground surface, respectively. Benzene concentrations (not shown) were very low. The shallow TPH concentrations may have been due to volatile organic compounds (VOCs) other than BTEX compounds.

Figure: indent 2.54 cm if less than 12.2 cm wide

Place figures between 12.2 and 14.8 cm wide at left-hand margin

Centre figures between and 14.8 and 17 cm wide

Check all figures are legible! The labelling must be larger to read!

Crop unnecessary space around figures

Remove borders from graphs

CALCULATION OF BIODEGRADATION RATE ESTIMATES

Figure caption: 10 pt; fully justify; indent 2.54 cm; put “Fig. 1” in bold

Leave 0.75 cm between caption and text

Fig. 1 Depth profile of residual hydrocarbon concentrations in a soil core.If the zone of oxygen and VOC consumption is restricted to the top shallow depth of the vadose zone, and diffusion is the only transport mechanism for oxygen through the vadose zone, then the governing equation is:

c(z)=c0−(c0+AL2)zL+Az2	(1)

Equation: indent 0.7 cm; put equation number in parentheses at right-hand margin; skip 1 line above and below

where c(z) is the oxygen concentration (%) at depth z (m), D is the effective diffusion coefficient for oxygen through the pore space of the vadose zone (m2 s-1),  is the zero-order oxygen consumption rate (% s-1), and A = /(2D). Note that c0 is the oxygen concentration at the ground surface, i.e. c(z) = c0 at z = 0, and L is the depth at which the oxygen concentration is zero, i.e. c(z) = 0 at z = L. To estimate , the depth z0 at which the oxygen concentration drops to half of the atmospheric concentration [c(z0) = c0/2] is substituted into equation (1) giving:

Put variables in both equations and text in italic

g=2Dz0Lc0(0.5−z0L)(1−z0L)	(2)

For all calculations, c0 was taken as 20%, and D was taken as 4–6×10-6 m2 s-1 for an air-filled porosity of 0.4 m3 m-3 (Glinski & Stepniewski, 1985). From three static depth profiles: L = 0.5 m, z0 = 0.15 m; L = 0.75 m, z0 = 0.35 m; L = 0.8, z0 = 0.2. These values gave  estimates of 3.3–80% day-1 with an average range (min.–max.) of 28–43% day-1. These oxygen consumption rates are high, but comparable to rates obtained by Newman & Barr (1997) and Davis et al. (1995). The average oxygen consumption rates gave hydrocarbon degradation rates of 110–170 mg-hexane l-1 day-1. Carbon dioxide concentration profiles gave comparable but lower rates.

Fig. 2 O2, CO2 and total petroleum hydrocarbon (TPH) vapour concentrations.CONCLUSIONS

Soil gas profiles measured at approximately steady state indicated that oxygen consumption, carbon dioxide production and biodegradation of hydrocarbon vapours occurred in a shallow and thin depth-interval of the vadose zone. Zero-order oxygen consumption rate coefficients and apparent hydrocarbon biodegradation rates calculated from the steady state profiles were high. Several issues remain including: the characteristic times to achieve steady state, diurnal and other oxygen concentration fluctuations at shallow depths, and the role of other oxidisable organic matter. Further monitoring and field experiments are planned to investigate these issues and improve estimation of the biodegradation rates reported here.

(Skip 2 lines)

Acknowledgements This work was partly funded by BP. The assistance and support of Colin Johnston, John Rayner and others in the CSIRO Laboratory are acknowledged.

Acknowledgement(s): 12 pt; fully justify; word “Acknowledgement” in bold

(Skip 2 lines unless the section ends at the bottom of a page)

REFERENCES

Davis, G. B., Johnston, C. D., Patterson, B. M., Barber, C., Bennett, M., Sheehy, A. & Dunbavan, M. (1995) Monitoring bioremediation of weathered diesel NAPL using oxygen depletion profiles. In: Monitoring and Verification of Bioremediation (ed. by R. E. Hinchee, G. S. Douglas & S. K. Ong), 193–201. Bioremediation 3(5).

Davis, G. B., Power, T. R., Briegel, D., Coad, D., Patterson, B. M. & Franzmann, P. D. (1997) Biodegradation of BTEX vapours in the vadose zone: steady state observations and results of an initial field experiment at BP Refinery Kwinana. Interim Report to BP CoE. CSIRO Land and Water Consultancy Report no. 97-54.

References: 8½ pt; 0.7 cm hanging indent; entries listed alphabetically;

Journal titles and book titles in italic;

volume numbers of journals in bold.

Note there is a space between initials

Glinski, J. & Stepniewski, W. (1985) Soil Aeration and Its Role for Plants. CRC Press Inc, Boca Raton, Florida, USA.

Johnston, C. D., Patterson, B. M., Rayner, J. L. & Davis, G. B. (1997) Evaluation of air sparging for biodegradation and physical removal of gasoline components from groundwater. Trial at the BP Kwinana Refinery, December 1996, CSIRO Land and Water Consultancy Report no. 97–6.

Newman, W. A. & Barr, K. D. (1997) Assessment of natural rates of unsaturated zone hydrocarbon bioattenuation. In: In Situ and On-Site Bioremediation (ed. by B. C. Alleman & A. Leeson), 1–6. Bioremediation 4(1).

Please use English British spelling; e.g.

- use groundwater (instead of ground water or ground-water)

- use modelling and modeller (instead of modeling and modeler)

- use colour, vapour, favour (instead of colour, vapor and favor)

Insert Figures and Tables soon after they are mentioned in the text